Digital CMOS VLSI Design

Fall 2003
Handout on Hspice

Introduction to Hspice ; What is Hspice. Whats its advantages over Verilog…….(1 paragraph).[iam preparing it]
Consider a Spice file.(say a inverter file named inv.sp)

1.  
*Inverter Circuit
2. 
m0 out in Vdd Vdd pfet w=4U l=2U 

3. 
m1 out in GND Gnd nfet w=2U l=2U 

4.  
CLOAD out 0 1pF 

5.  
Vdd Vdd 0 5
6.  
Vin in 0 0 PULSE .2 4.8 2N 1N 1N 5N 20N
7.  
.OPTIONS LIST NODE POST 

8.  
.TRAN 200P 20N 

9.  
.PRINT TRAN V (in) V(out) 

10. 
.MEASURE avgpow avg power from 0.00ns to 10.00ns 

11. 
.MODEL pfet PMOS LEVEL=1 

12. 
.MODEL nfet NMOS LEVEL=1 

13. 
.END 1 


Note : The comment must start with *

[image: image1.png]GND










Fig.1a

Input Description 

Any spice input file (with an extension .sp) contains four types of statements: 

1. Data statements: description of the components and the interconnections (Lines 2-6 in the inv.sp description). 

2. Control statements:tells SPICE what type of analysis to perform on the circuit(Lines 7-8).

3. Output statements: specifies what outputs are to be printed or plotted (Lines 9-10).

4. Model statements: specifies what models need to be used for the simulation of components (eg. FETs) (Lines 11-12). 

The textual description in the previous page models the inverter shown in Figure 1(a). The first line always contains the description of the circuit. Line 2 describes the PFET and line 3, the NFET. The general format of FET description is: 

Mname ND NG NS < NB > ModName W= L=

The MOS transistor name (Mname) has to start with a M; ND, NG, NS and NB are the node numbers of the Drain, Gate, Source and Bulk terminals, respectively. By default, the node number 0 represents the GND node. ModName is the name of the transistor model (see further). L and W are the length and width of the gate (in m). 

Line 4 describes a capacitor. General format of a capacitor is: 

Cname N1 N2 Value < IC >

Where N1 is the positive node, N2 is the negative node. IC is the initial condition (DC voltage or current). The symbol <> means that the field is optional. If not specified, it is assumed to be zero. Lines 5 and 6 describe voltage sources. General format for independent voltage and current sources is: 

Voltage source: Vname Name N1 N2 Type Value 

Current source: Iname Name N1 N2 Type Value2 

Where N1 is the positive terminal node, N2 is the negative terminal node. Type can be DC, AC or TRAN, depending on the type of analysis. Value gives the value of the source. The name of a voltage and current source must start with V and I respectively. 

Line 6 describes a voltage pulse source. The general format of a PULSE description (see Figure 1(b)) is:

[image: image2.png]







Fig.1b

Vname N1 N2 PULSE V1 V2 TD Tr Tf  PW Period

Where V1 - initial voltage; V2 - peak voltage; TD - initial delay time; Tr - rise time; Tf - fall time; PW - pulse-width; and Period - total time period.

Line 7 specifies various options that we would like to use for simulation. The argument "LIST" produces an element summary of list of the input data to be printed. The argument "NODE" causes a node cross-reference table to be printed. The argument "POST" enables storing of simulation results for analysis. 
Line 8 .TRAN asks the SPICE to perform a transient analysis from 0ns to 20ns in increments of 200 picoseconds. .PRINT statement on line 9 causes the voltage waveforms to be printed. .MEASURE statement on line 10 specifies the circuit properties we would like measure in this case the average power consumption in the time period 0ns to 10ns. Lines 11-12 specify the models we want to use for the FETs in the circuit. LEVEL=1is the basic model that incorporates only first-order effects of the MOSFET. The last line in the input description is an .END line that signals the end of the circuit input.

Simulating the Circuit in awaves

1. In the AvanWaves window, select Design ( Open ( Filter ( Input. This will list the input file "inv.sp" in the dialog box. Press Ok. 

2.  Select Design ( Current. This will invoke a dialog box entitled "Current Design". Click on the item D0: <../..> /inverter and press OK. Now the current design is inv.sp.

3. Select Tools ( Run HSPICE. This will invoke a "Run Manager" window. Select the design and click on the Run button. In the left sub window, the status of the design changes from Running to Done at the end of the simulation. 

4.  Select Tools ( Results Browser. In Results Browser window, Click on item "Transient: inverter circuit" in the top dialog box. This will give rise to various fields in "Types:" and "Curves:" dialog boxes. 

5. In the "Curves:" dialog box, double click on V(in) and V(out). This will result in a display of the voltage waveforms on the nodes in and out.

6. Familiarize yourself with various options available under the buttons: Panels, Window, Measure, and Configuration. 

7. Select Tools ( Print. Select File and Postscript options in the dialog box. Type a file name, say, inv.ps. Click Print. This will save the current waveform window as a postscript file. 

Running from the command line: 

If you wish to run HSpice from the command line, then type: 

Sun Server: hspice inv.sp > inv.lis
When the job finishes, HSPICE displays: 

>info: 


***** hspice job concluded  

real 1.1

user 0.1 

sys 0.1 

When you are done with the simulation, you can check for correct operation of the circuit by looking at the output waveforms that can be plotting in awaves by typing this command at the sun server prompt within the invsp.run1 directory(if you are checking the inverter circuit).

Sun Server: awaves inv ; the windows shown in Fig.2 and Fig.3 pop up

                             [image: image3.png]=181

Design: [D0: momefgradandralspice.runi invert

ol Feasient:  neto - foutput =
7
& -
o e Gurves:
frme z 5 |
(urents J
7|
Curont -fsis
TIME [
Fitor

E Apply. Default
Close Help





 Fig.2

[image: image4.png]| AvanWaves 2001.4 (20011215)

Design Panels Window Measure Configuration Tools

Voltages (iin)

150n
Tie (in) (TME)





   Fig.3

You can now plot the required signal on the awaves window. You can analyze the results by viewing the "inv.lis" and "inv.st0" files in your favorite text editor! 

When you spice the input file, the following new files will be in your directory: inv.lis, inv.ic, inv.st0, and inv.mt0. Familiarize yourself with the contents of these files. 
SPICE Manuals and Resources:
An online version is available at: 

http://www-scf.usc.edu/~ee577/cad_tools.html

Simulating Cadence Virtuoso Layout in Hspice

Open the Extracted view and select Tools ( Other

Then select simulation ( Initialize the following window (Fig.4) pops up, select the run directory as say spice.run1. 

[image: image5.png]ok | cancel | Derauts| appty Help

‘Simulation Run Directory | /home/grad/nandray/spice. runl]





Fig.4

Click "OK", another window pops up, select the simulator name as "hspice" and click "OK".

Now select Simulation ( Options, a window shown in the Fig.5 pops up, select "Re-netlist Entire Design" and click "OK".

[image: image6.png]ulation Environment Options:

oK | cocot | Dotauts| sy | vt
Use Hierarchical Netlister =}

Incremental Netiisting r

Re-netlist Entire Design -
Run Simulation Remotely r

Simiation tost e [Losalhost

Simulation Host s Different Type |





Fig. 5
Select Simulation ( Netlist/Simulate which will lead to the popup window shown below in Fig.7, uncheck the simulate option and click "OK".

[image: image7.png]Netlist and Simulate
oK | cocot | potauts| sy | ot
Stmidation fas Directory [ onefgead foandea apise vund
= T
(o
Cell Name |mven£
View Name [extracted
Simulator Name: hspice I
Run Actions. W netlist _| simulate
Run in Background - 10

Jobpriorty @-20) | 11





Fig. 6
The following window showing the successful creation of the netlist pops up (Fig.7).

[image: image8.png]Analysis Job Succeeded

§ 30b */hone/grad/nandra/spice. runl” that vas started at ‘Sep 12 15:33:50 2002 has succesded





Fig.7
Now go to the sun server terminal change the director to the "spice.run1" directory. The directory has a file named "netlist" that contains the following lines.

* net 0 = /output

* net 1 = /vdd

* net 2 = /inp

* net 3 = /gnd

.model model1 pmos level=2 vto=-0.7 gamma=0.4 kp=1.5e-05

+lambda=0.03 tox=6e-07

* pmos(0) = /+0

m0 0 2 1  model1 w=1.45u l=0.45u

.model model2 nmos level=2 vto=0.7 gamma=0.2 kp=3e-05

+lambda=0.02 tox=6e-07

* nmos(1) = /+1

m1 0 2 3  model2 w=1.45u l=0.45u

Change the name of the netlist file as invert.sp. Edit this file by adding the desired statements. 

* net 0 = /output

* net 1 = /vdd

* net 2 = /inp

* net 3 = /gnd

vvdd 1 3 dc 5v

vinp 2 3 dc 5v

.model model1 pmos level=2 vto=-0.7 gamma=0.4 kp=1.5e-05

+lambda=0.03 tox=6e-07

* pmos(0) = /+0

m0 0 2 1 1 model1 w=1.45u l=0.45u

.model model2 nmos level=2 vto=0.7 gamma=0.2 kp=3e-05

+lambda=0.02 tox=6e-07

* nmos(1) = /+1

m1 0 2 3 3  model2 w=1.45u l=0.45u

.options post list

.tran 0.1ns 300ns

.end

*note: The lines in bold are the newly added ones.

The edited file might be simulated by following the procedure given on pages 3 and 4.

PAGE  
5

