The Tennessee Eastman Project

#### Creation of an S-Function to Model the TE Process

Nick Graham

Andrew Tillinghast

ChE 462

Instructor Babu Joseph

#### April 28, 2000

# I.   What We Accomplished

Our main accomplishment is the implementation of a working S-function that models the TE process.  We went through the following steps to achieve our goal:

- Gained an understanding of the S-function block in Simulink

- Stepped manually through the ‘tefunc’ program to understand how it worked

- Set-up a Simulink model to run the ‘tefunc’ & ‘teinit’ programs

- Wrote an S-function to use the ‘tefunc’ and ‘teinit’ programs so that they correctly model the TE problem in Simulink.

- Implemented internal controllers with the following parameters to stabilize the process in the absence of disturbances:

Table 1.  Internal Controllers and their parameters

- Updated the user interface with external controllers using the following parameters to stabilize the reactor, stripper, and separator levels in the presence of a disturbance:

Table 2. External Controllers and their parameters

# Tests were run to examine the behavior of all Simulink models under different operating conditions.

# II. Test Results

Four variables that were closely monitored: the levels of the reactor, the separator and the stripper, and the reactor pressure.  It was found that when no controllers  (external or internal) existed, the process ran out of control as seen in Figure 1 below:

Figure 1: The TE Process without Controllers

When the internal controllers were added, the process exhibited a steady state as seen below in Figure 2:

Figure 2: The TE Process with Internal Controllers

After implementing the external controllers, the performance of the model without external controllers and the one with external controllers in the presence of a disturbance was tested.  The A Feed loss disturbance (Disturbance #6, see Appendix) was stepped from 0 to a value of 10 at 500 seconds; this disturbance was inputted to Simulink models Two4_new (without external controllers) and Two5 (with external controllers) (see Appendix for Simulink Diagrams).

The model without external controllers was clearly out of control as the levels deviated from their steady state values and no corrective action was observed.  The model with external level controllers, however, kept the levels near their steady state values at all times.  A comparison of these simulations is shown in Figure 3.

Figure 3: The TE Process in the Presence of an A Feed Loss

Without External Controllers					With External Controllers

For the model with external level controllers, however, the reactor pressure did not remain at steady state since there was no controller assigned to this variable.  Thus, although the levels are clearly in control, other variables in the TE plant are still sensitive to disturbances.  The graph of reactor pressure versus time is shown in Figure 4.

Figure 4: Reactor Pressure vs. Time for Two5.mdl

# III.  Issues Resolved

# We consider the following issues to be resolved:

# The creation of a working S-Function that models that TE process.

# The stabilization of the TE Process under normal operating conditions using internal controllers.

# The stabilization of the reactor, separator, and stripper levels in the presence of a disturbance using external controllers.  Please note that the reactor pressure, among other variables, is not necessarily stable in the presence of a prolonged disturbance.

# IV.    Appendix

### S-Functions

We have appended two S-Function programs:

- sfnc3.m – the TE Plant with no internal controllers

- sfnc5.m – the TE Plant with internal controllers for the manipulated variables

Both functions initialize the TE plant using the MEX file TEINIT and then step the plant through time by calling TEFUNC.

Both programs require a step size T_samp plus 32 inputs.  In order to specify T_samp, one must open the S-Function Simulink block and enter the value under “S-Function parameters.”

For sfnc3.m, the first 12 values of the additional 32 inputs correspond to the actual values of the manipulated variables.  These inputs specify the xmv array used by TEFUNC.  All values except for the agitator speed are listed as percentages:

| Input Number | Manipulated Variable | Base Case Values |
| --- | --- | --- |
| 1 | D Feed Flow Valve | 63.053 |
| 2 | E Feed Flow Valve | 53.980 |
| 3 | A Feed Flow Valve | 24.644 |
| 4 | A/C Feed Flow Valve | 61.302 |
| 5 | Compressor Recycle Valve | 22.210 |
| 6 | Purge Valve | 40.064 |
| 7 | Separator Liquid Underflow Valve | 38.100 |
| 8 | Stripper Liquid Underflow Valve | 46.534 |
| 9 | Stripper Steam Valve | 47.446 |
| 10 | Reactor Cooling Water Valve | 41.106 |
| 11 | Condenser Cooling Water Valve | 18.144 |
| 12 | Agitator Speed | 50.000 rpm |

The last 20 input variables correspond to the disturbances entering the process.  Under normal conditions, these variables are set to zero.  These disturbances must be specified in a vector named “disturb” in the Matlab workspace.  Curiously, the vector “disturb” must have a length of 21 in order for the process to work.  For an unknown reason, the “From Workspace” box in the Simulink model transfers only variables 2 to 21 into the model.  We usually declared this array using the Matlab command:

disturb=zeros(1,21)

The descriptions corresponding to these variables are listed on the next page.

| Disturbance Number | Type | Description |
| --- | --- | --- |
| 1 | Step | A/C Feed Ratio |
| 2 | Step | B composition |
| 3 | Step | D Feed temperature |
| 4 | Step | Reactor Cooling Water Inlet Temperature |
| 5 | Step | Condenser Cooling Water Inlet Temperature |
| 6 | Step | A Feed loss |
| 7 | Step | C Header Pressure loss |
| 8 | Random Variation | A, B, C Feed composition |
| 9 | Random Variation | D Feed Temperature |
| 10 | Random Variation | C Feed Temperature |
| 11 | Random Variation | Reactor Cooling Water Inlet Temperature |
| 12 | Random Variation | Condenser Cooling Water Inlet Temperature |
| 13 | Slow Drift | Reaction Kinetics |
| 14 | Sticking | Reactor Cooling Water Valve |
| 15 | Sticking | Condenser Cooling Water Valve |
| 16 | Unknown | Unknown |
| 17 | Unknown | Unknown |
| 18 | Unknown | Unknown |
| 19 | Unknown | Unknown |
| 20 | Unknown | Unknown |

Regarding the Matlab vector, disturb(2) = Disturbance #1 (A/C Feed Ratio).

For sfnc5.m, the first 12 values of the additional 32 inputs correspond to the setpoints for the manipulated variables.  Please note that these are setpoint values and not the actual values of the manipulated variables.

| Input Number | Manipulated Variable | Setpoint for Base Case |
| --- | --- | --- |
| 1 | D Feed Flow | 3664 kg/h |
| 2 | E Feed Flow | 4509.3 kg/h |
| 3 | A Feed Flow | 0.25052 kscmh |
| 4 | A/C Feed Flow | 9.3477 kscmh |
| 5 | Compressor Recycle Setpoint | 22.210 % |
| 6 | Purge Flow | 0.33172 kscmh |
| 7 | Separator Liquid Underflow | 25.160 m3/h |
| 8 | Stripper Liquid Underflow | 22.949 m3/h |
| 9 | Stripper Steam Flow | 230.31 kg/h |
| 10 | Reactor Cooling Water Temperature | 94.599 C |
| 11 | Condenser Cooling Water Temperature | 77.297 C |
| 12 | Agitator Speed | 50 rpm |

### Simulink Diagrams

We have appended three Simulink Diagrams:

### Two4.mdl – uses sfnc3.m and contains neither internal nor external controllers

- Two4_new.mdl – uses sfnc5.m and contains internal but no external controllers

- Two5.mdl – uses sfnc5.m and contains both internal and external controllers

We recommend that a T_samp of 1 second be used for all of these Simulink models.

The inputs for Two4.mdl are the actual values for the manipulated variables.  These are given in the table above detailing the inputs to sfnc3.m.

The inputs for Two4_new.mdl are the setpoints for the manipulated variables.  These setpoints are given in the table above detailing the inputs to sfnc5.m.

The majority of the inputs for Two5.mdl are identical to those for Two4_new.mdl; however, Two5.mdl also contains external controllers.  These controllers adjust the setpoints of the manipulated variables based on measurements of the reactor level, the separator level, and the stripper level in order to control these same variables.  The following external PI controllers have been added:

| Manipulated Variable | Measured and Controlled Variable | Setpoint of Measured Variable (%) |
| --- | --- | --- |
| Setpoint of E Feed | Reactor Level | 75 |
| Setpoint of Separator Underflow | Separator Level | 50 |
| Setpoint of Stripper Underflow | Stripper Level | 50 |

Thus, the outputs of these PI controllers take the place of the constant blocks for E Feed Setpoint, Separator Underflow Setpoint, and Stripper Underflow Setpoint inputs.

The PI controllers used in the Simulink Blocks were programmed by Babu Joseph.